UNIVERSITY “POLITEHNICA” of BUCHAREST

Faculty of Electronics, Telecommunications and Information Technology

CAD Project

Solar-Powered Plant Alarm

Dorca Victor

Simionescu David-Daniel

Coordinator: Prof. Dr. Ing. Pantazică Mihaela

Group 424Ga

CONTENTS

I. Initial design data................................................................................................................. 1

II. Description of the design schematic functionality.......................................................... 2

III. Electrical schematic............................................................................................................ 3

IV. Design Rules check (DRC).................................................................................................. 4

V. Cross Reference (CR)........................................................................................................... 5

VI. Bill of materials (BOM)....................................................................................................... 6

VII. Wirelist (WR)....................................................................................................................... 7

VIII. Nets verification............................................................................................................... 9

IX. Layer Copper TOP............................................................................................................. 10

X. Layer Copper BOTTOM...................................................................................................... 11

XI. Layer Solder Mask TOP.................................................................................................... 12

XII. Layer Solder Mask BOTTOM.......................................................................................... 13

XIII. Layer silk screen TOP..................................................................................................... 14

XIV. Layer Assembly Drawing TOP....................................................................................... 15

XV. Layer Fabrication............................................................................................................. 16

XVI. Conclusions..................................................................................................................... 17

XVII. Bibliography................................................................................................................... 18

XVIII. ANEXES.......................................................................................................................... 19

SOLAR POWERED PLANT ALARM

Initial Design Data

The primary objective of this project is to develop a PCB design for a solar-powered soil moisture warning system that operates entirely without the need for external batteries. This electronic device is specifically designed to monitor the electrical resistance of the soil through two metal probes and to activate an acoustic alarm whenever the moisture level drops below a certain threshold. The system is powered by a solar cell that provides a nominal voltage of 6V which is sufficient to drive the entire circuit during daylight hours. For visual confirmation of the power status, the design incorporates two LEDs connected in series which serve as an indicator that the solar panel is generating at least 4V to ensure proper operation. The circuit utilizes NPN transistors such as the BC547 and 2N2222 to handle signal amplification and logic inversion while a 6V piezoelectric buzzer is used as the final output for the audible alert. Precision in detection is achieved through a 50kΩ or 100kΩ potentiometer that allows the user to calibrate the sensitivity of the sensors according to the specific needs of the plant or the type of soil used.

Description of the Designed Schematic Functionality

The functionality of the schematic is based on the interaction between a moisture sensor and a transistor-based logic gate that triggers an alarm under dry conditions. The two metal probes inserted into the ground act as a variable resistor whose value changes significantly depending on the water content of the soil. When the solar cell is exposed to light the current flows through a current-limiting resistor and the two series LEDs to provide a visual status of the power supply. If the soil is sufficiently wet the low resistance between the probes allows current to flow directly to the base of the first transistor which turns it on and effectively pulls the base of the second transistor to ground. In this state the second transistor remains off and the buzzer is silent because the path to the negative terminal is disconnected. However, when the soil dries out the resistance between the probes increases to a point where the first transistor can no longer stay active. This change causes the signal to be inverted through the common-emitter configuration which then allows the second transistor to turn on and complete the circuit for the piezoelectric buzzer. The specific threshold for this activation is determined by the setting of the potentiometer which modifies the voltage divider ratio and ensures that the alarm only sounds when the moisture level is truly inadequate.

Electrical Schematic

Design Rules Check (DRC)

--------------------------------------------------

Checking Schematic: SCHEMATIC1

--------------------------------------------------

Checking Electrical Rules

Checking For Single Node Nets

Checking For Unconnected Bus Nets

Cross Reference (CR)

Cross Reference            Page1

Item    Part    Reference    SchematicName    Sheet    Library

____________________________________________________________________________

1    2N3904    Q1    SCHEMATIC1/PAGE1    1    D:\TCAD\TOOLS\CAPTURE\LIBRARY\TRANSISTOR.OLB

2    2N3904    Q2    SCHEMATIC1/PAGE1    1    D:\TCAD\TOOLS\CAPTURE\LIBRARY\TRANSISTOR.OLB

3    10k    R1    SCHEMATIC1/PAGE1    1    D:\TCAD\TOOLS\CAPTURE\LIBRARY\DISCRETE.OLB

4    50k    VR1    SCHEMATIC1/PAGE1    1    D:\TCAD\TOOLS\CAPTURE\LIBRARY\DISCRETE.OLB

5    100    R2    SCHEMATIC1/PAGE1    1    D:\TCAD\TOOLS\CAPTURE\LIBRARY\DISCRETE.OLB

6    CEM-1606    LS1    SCHEMATIC1/PAGE1    1    C:\USERS\VDORC\DOWNLOADS\CEM-1606\CEM-1606.OLB

7    LED    D1    SCHEMATIC1/PAGE1    1    D:\TCAD\TOOLS\CAPTURE\LIBRARY\DISCRETE.OLB

8    LED    D2    SCHEMATIC1/PAGE1    1    D:\TCAD\TOOLS\CAPTURE\LIBRARY\DISCRETE.OLB

9    Probes    J2    SCHEMATIC1/PAGE1    1    D:\TCAD\TOOLS\CAPTURE\LIBRARY\CONNECTOR.OLB

10    Solar cell    J1    SCHEMATIC1/PAGE1    1    D:\TCAD\TOOLS\CAPTURE\LIBRARY\CONNECTOR.OLB

Bill of Materials (BOM)

| No. | Qty. | Reference | Component | Description | Mount | Capsule | Manuf. | Distributor | Code product | Price / Qty. (RON) | Min. Qty. | Order’s Price |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | D1, D2 | LED | Single Colour LEDs Single LED 3mm Red 20mA 2VDC | THT | CAP196 | EAO | ro.mouser.com | 123-10-2601.3172S | 35,31 | 1 | 70,62 |
| 2 | 1 | J1 | Solar cell | Solar Panels & Solar Cells Monocrystalline Solar Cell, 100mA, 3.6V | THT | JUMPER2 | OSEPP Electronics | ro.mouser.com | 375-SC10036 | 33,25 | 1 | 33,25 |
| 3 | 1 | J2 | Probes | Sensor: humidity; for soil,capacitive; analogic; 3,3÷5VDC | THT | JUMPER2 | RFROBOT | www.TME.EU | DF-SEN0114 | 21,93 | 1 | 21,93 |
| 4 | 1 | LS1 | CEM-1606 | Buzzers & Audio Indicators - Board Mount Buzzers | THT | CUI_CEM-1606 | Same Sky | ro.mouser.com | 490-CEM-1606 | 5,89 | 1 | 5,89 |
| 5 | 2 | Q1, Q2 | 2N3904 | Bipolar Transistors - BJT BJT, TO-92, 40V, 200mA, NPN | THT | TO92 | Diotec Semiconductors | ro.mouser.com | 637-2N3904 | 0,52 | 1 | 1,04 |
| 6 | 1 | R1 | 10k | Carbon Film Resistors - Through Hole 1 kOhms 250 mW 5% -350/ +500 PPM / C | THT | RES400 | YAGEO | ro.mouser.com | 603-CFR-25JT-52-1K | 0,43 | 1 | 0,43 |
| 7 | 1 | R2 | 100 | Carbon Film Resistors - Through Hole 100ohm 1/4W 5% | THT | RES401 | YAGEO | ro.mouser.com | 603-CFR-25JR-52100R | 0,43 | 1 | 0,43 |
| 8 | 1 | VR1 | 50k | 1/4inch square potentiometer, Side or top adjustment with lead screw, Resistance value ranges from 20 ohm to 2Mohm | THT | VRES58 | BOURNS | ro.farnell.com | 9354387 | 4,2 | 1 | 4,2 |

Wirelist (WR)

Wire List

<<< Component List >>>

LED                           D1        CAP196

LED                           D2        CAP196

Solar cell                    J1        JUMPER2

Probes                        J2        JUMPER2

CEM-1606                 LS1       CUI_CEM-1606

2N3904                      Q1        TO92

2N3904                      Q2        TO92

10k                             R1        RES400

100                             R2        RES400

50k                            VR1       VRES58

<<< Wire List >>>

NODE REFERENCE    PIN #   PIN NAME           PIN TYPE       PART VALUE

[00001] GND

VR1             1       A               Passive        50k

Q1                1       EMITTER         Passive        2N3904

D2                2       CATHODE         Passive        LED

J1                 2       2               Passive        Solar cell

Q2                1       EMITTER         Passive        2N3904

[00002] N00346

D1              1       ANODE           Passive        LED

R2              2       2               Passive        100

[00003] N00350

D1              2       CATHODE         Passive        LED

D2              1       ANODE           Passive        LED

[00004] N00431

Q1              3       COLLECTOR       Passive        2N3904

R1              2       2               Passive        10k

Q2              2       BASE            Input          2N3904

[00005] N00435

R2              1       1               Passive        100

J1               1       1               Passive        Solar cell

J2               2       2               Passive        Probes

R1              1       1               Passive        10k

LS1            P       P               Passive        CEM-1606

[00006] N00515

LS1             N       N               Passive        CEM-1606

Q2               3       COLLECTOR       Passive        2N3904

[00007] N01959

VR1             3       B               Passive        50k

VR1             2       WIPER           Passive        50k

Q1               2       BASE            Input          2N3904

J2                1       1               Passive        Probes

Nets Verification

Layer Copper Top

Facultatea de Electronica, Telecomunicatii si Tehnologia Informatiei - UPB

Proiect CAD “Solar-Powered Plant Alarm”

Coordonator: Prof. Dr. Ing. Mihaela Pantazica

Studenti: Dorca Victor, Simionescu David-Daniel 424G

Layer Copper Bottom

Facultatea de Electronica, Telecomunicatii si Tehnologia Informatiei - UPB

Proiect CAD “Solar-Powered Plant Alarm”

Coordonator: Prof. Dr. Ing. Mihaela Pantazica

Studenti: Dorca Victor, Simionescu David-Daniel 424G

Layer Solder Mask Top

Facultatea de Electronica, Telecomunicatii si Tehnologia Informatiei - UPB

Proiect CAD “Solar-Powered Plant Alarm”

Coordonator: Prof. Dr. Ing. Mihaela Pantazica

Studenti: Dorca Victor, Simionescu David-Daniel 424G

Layer Solder Mask Bottom

Facultatea de Electronica, Telecomunicatii si Tehnologia Informatiei - UPB

Proiect CAD “Solar-Powered Plant Alarm”

Coordonator: Prof. Dr. Ing. Mihaela Pantazica

Studenti: Dorca Victor, Simionescu David-Daniel 424G

Layer Silkscreen

Facultatea de Electronica, Telecomunicatii si Tehnologia Informatiei - UPB

Proiect CAD “Solar-Powered Plant Alarm”

Coordonator: Prof. Dr. Ing. Mihaela Pantazica

Studenti: Dorca Victor, Simionescu David-Daniel 424G

Layer Assembly Drawing

Facultatea de Electronica, Telecomunicatii si Tehnologia Informatiei - UPB

Proiect CAD “Solar-Powered Plant Alarm”

Coordonator: Prof. Dr. Ing. Mihaela Pantazica

Studenti: Dorca Victor, Simionescu David-Daniel 424G

Layer Fabrication

Facultatea de Electronica, Telecomunicatii si Tehnologia Informatiei - UPB

Proiect CAD “Solar-Powered Plant Alarm”

Coordonator: Prof. Dr. Ing. Mihaela Pantazica

Studenti: Dorca Victor, Simionescu David-Daniel 424G

Conclusions

The development of the Solar Plant Watering Alarm project illustrates a successful integration of simple electronic principles to solve a practical everyday problem in a sustainable manner. By utilizing solar energy, the design eliminates the ongoing costs and environmental waste associated with disposable batteries while maintaining a high degree of reliability during the day. The use of basic components like transistors and resistors to create a logic inverter proves that complex integrated circuits are not always necessary to achieve effective automation in gardening applications.

This project not only provides a functional tool for plant maintenance but also serves as a lesson for understanding how sensors and renewable energy sources can be combined into a robust PCB design. Ultimately the clarity and simplicity of this circuit ensure that it can be easily manufactured and adjusted to suit a wide variety of environmental conditions and user requirements.

Bibliography/ Webography

- https://www.cadence.com/en_US/home/tools/pcb-design-and-analysis/orcad.html

- https://pcbsync.com/how-to-generate-gerber-files-from-cadence-allegro/

- https://tracespace.io/view/

- https://ro.mouser.com/

- https://www.tme.eu/ro/

- https://www.youtube.com/user/kirschmackey/videos

ANNEX 1

Solar-powered Plant watering alarm circuit

This solar plant watering alarm circuit will sound an alarm when your plants are drying out, letting you know when to water them. It is self-sustainable because it is powered by a solar cell.

I always teach my children how to plant a tree, which typically begins with a vegetable in small pots. Even if we used the best fertilizer, if we forgot to water the plants, they would have died in no time.

On the other hand, if we water the plants too much, it may cause their roots to rot or cause other diseases. Thus, maintaining the soil’s humidity is very important for our plants.

That’s why we are building this solar plant watering alarm circuit, or soil humidity alarm circuit, with the following features:

1. A buzzer alarms when the soil’s humidity decreases.

2. Be as energy-efficient as possible.

3. Only sounds an alarm during the day; if it sounds at night, it can be disruptive to our neighbor.

4. Uses low-cost and basic components.

5. A simple circuit that is suitable for learning basic electronics.

ANNEX 2

ANNEX 3

ANNEX 4